CAPSTONE LOCALIZATION

ESP32 ToF Subsystem Refactoring

Architecture Strategy & Implementation Plan

Decisions: USB-serial flashing | Thin-tag architecture | Recompile-per-device config

# 1. Executive Summary

This document defines the refactoring strategy for absorbing the ESP32-S3 Time-of-Flight (ToF) subsystem into the main Hybrid localization engine. The goal is to turn each ESP device into a thin client that streams raw FTM ranging data over MQTT, while the engine performs all positioning math (trilateration, GP regression, fingerprinting). A deployment module in the platform backend enables build-and-flash of ESP firmware directly from the UI.

## Current vs. Target Architecture

| Aspect | Current State | Target State |
| --- | --- | --- |
| Tag role | Smart: runs local trilateration, publishes computed (x,y) | Thin: publishes raw per-anchor distances only |
| Anchor role | Publishes raw ranges (already thin) | No change needed |
| MQTT topics | tof/anchor/<id>/range, tof/tag/<id>/position | capstone/tof/<device_type>/<id>/range (unified) |
| Payload format | CSV (12 fields, ms-since-boot timestamps) | JSON (ISO-8601 timestamps, metric units) |
| Config method | #define hardcoded per device | Auto-generated device_config.h, platform triggers idf.py build+flash |
| Positioning math | Duplicated: tag firmware + dashboard app.py + Hybrid engine | Single source: Hybrid engine only |
| Dashboard | Standalone Flask/SocketIO app | Retired; platform frontend handles visualization |

# 2. Architecture Refactoring

## 2.1 Responsibility Separation

The refactoring draws a clear line: hardware interaction stays on the ESP, everything else moves to the engine.

| Stays on ESP (Firmware) | Moves to Engine (Python) |
| --- | --- |
| WiFi AP+STA initialization | Trilateration (least-squares solver) |
| FTM responder mode (SoftAP) | Confidence scoring algorithms |
| WiFi scan for peer SSIDs | Position smoothing / filtering |
| FTM session initiation + RTT extraction | Room classification (point-in-polygon) |
| RSSI capture during scan | Anchor position lookup table |
| MQTT connection + raw data publish | Multi-method fusion (ToF + RSSI + GP) |
| NVS flash init, watchdog | CSV/S3 logging, live map state |

## 2.2 What Gets Deleted from Tag Firmware

The following code blocks are removed from tag_main.c:

| Function / Block | Lines | Reason |
| --- | --- | --- |
| trilaterate() | 508-562 | Positioning math moves to engine |
| ANCHOR_POSITIONS[] lookup table | 72-85 | Engine owns anchor coordinates via config.yaml |
| get_anchor_position() | 388-398 | No longer needed without local trilateration |
| publish_position() | 254-311 | Replaced by publish_ranges() streaming raw data |
| Confidence calculation in publish_position | 272-279 | Engine computes confidence from raw measurements |
| send_to_server() (already disabled) | 567-623 | Dead code removal |

# 3. Data Format Migration

## 3.1 Unified MQTT Topic Scheme

All ToF devices publish under the capstone/ prefix to align with the Hybrid engine's existing MQTTPipe subscriber.

| Device | Old Topic | New Topic |
| --- | --- | --- |
| Anchor | tof/anchor/<id>/range | capstone/tof/anchor/<id>/range |
| Tag (raw ranges) | tof/tag/<id>/position | capstone/tof/tag/<id>/range |
| Config (future) | (none) | capstone/tof/<type>/<id>/config |

## 3.2 JSON Payload Format

Both anchors and tags publish the same JSON schema for range measurements. The engine distinguishes source by the topic path.

{
  "device_id": "anchor_4",
  "target_id": "tag_0",
  "distance_m": 2.50,
  "rtt_ps": 166667,
  "rssi": -65,
  "confidence": 0.85,
  "scan_number": 42,
  "ts": "2026-03-31T20:15:00Z"
}

## 3.3 Timestamp Strategy

ESP32-S3 has no RTC battery, so timestamps are milliseconds-since-boot. The engine stamps each received message with server-side UTC on arrival. The firmware includes its uptime_ms field for relative ordering and duplicate detection, but the authoritative timestamp is engine-assigned.

# 4. Tag Firmware: Thin Client Design

The refactored tag firmware retains exactly four responsibilities: (1) WiFi AP+STA setup with FTM responder, (2) periodic scan for TOF_ANCHOR_* SSIDs, (3) FTM ranging to each discovered anchor, and (4) publishing raw range measurements as JSON to MQTT.

## 4.1 New publish_ranges() Function

Instead of a single publish_position() call after trilateration, the tag publishes one JSON message per anchor it successfully ranged. This gives the engine full visibility into individual measurements for filtering, outlier rejection, and multi-method fusion.

## 4.2 Auto-Generated device_config.h

All per-device #define values move to a generated header file. The platform backend writes this file before triggering idf.py build. The tag_main.c source becomes identical across all tags — only the config header differs.

| Config Key | Type | Example | Source |
| --- | --- | --- | --- |
| TAG_NUM | int | 0 | Platform device registry |
| DEVICE_ID | string | "tag_0" | Derived from TAG_NUM |
| WIFI_STA_SSID | string | "EAP1250" | Platform network config |
| WIFI_STA_PASS | string | "" | Platform network config |
| MQTT_BROKER_URI | string | "mqtt://192.168.100.10:1883" | Platform MQTT config |
| TAG_AP_CHANNEL | int | 1 | Platform RF config |
| TAG_AP_IP_LAST | int | 100 | TAG_NUM + 100 |
| LOCATION_ID | string | "lab_1" | Platform location hierarchy |
| FLOOR_ID | string | "floor_1" | Platform location hierarchy |

# 5. Engine-Side ToF Ingestion

## 5.1 MQTTPipe Fixes

The existing MQTTPipe in data_pipes.py subscribes to capstone/<mac>/tof and expects JSON with a distance_m field. The fix updates the subscription topic to capstone/tof/+/+/range (wildcard for device type and ID) and parses the new unified JSON payload into ToFMeasurement objects.

## 5.2 ToF Trilateration in the Engine

The tag's trilaterate() C function is reimplemented in Python inside engine_wrappers.py as a ToFWrapper class. It buffers raw range measurements per tag, applies outlier filtering (reject distances > 3σ from rolling mean), then runs the same linearized least-squares solver. The result feeds into _store_decision() as tof_decisions with full (x, y) coordinates.

## 5.3 Multi-Method Fusion

With ToF producing (x, y, confidence) alongside trilateration, fingerprinting, and Sparse GP, the engine can fuse estimates using confidence-weighted averaging. Each method's confidence becomes a weight: x_fused = Σ(conf_i * x_i) / Σ(conf_i). This is implemented as a FusionWrapper that reads from all four position dicts and emits a single fused_positions entry per device.

# 6. In-App Flashing Module

## 6.1 Architecture

The flashing pipeline is: UI form → POST /api/v1/esp/flash → generate device_config.h → idf.py build → idf.py flash → stream build/flash logs back to UI via SSE (Server-Sent Events).

## 6.2 Backend: ESP Flash Router

A new FastAPI router (platform/backend/api/esp_flash.py) exposes:

| Endpoint | Method | Description |
| --- | --- | --- |
| /api/v1/esp/ports | GET | List available serial ports (pyserial) |
| /api/v1/esp/flash | POST | Trigger build+flash for a device config |
| /api/v1/esp/flash/status | GET (SSE) | Stream build/flash progress logs |
| /api/v1/esp/flash/cancel | POST | Cancel a running build/flash job |

## 6.3 Build Pipeline

The flash endpoint receives device parameters (type, ID, network config), writes a device_config.h into the firmware source tree, then spawns idf.py build and idf.py -p <port> flash as subprocesses. stdout/stderr are captured line-by-line and pushed to the SSE stream. The UI displays a live terminal-style log with progress indicators.

## 6.4 Prerequisites

The host machine running the platform must have: (1) ESP-IDF v5.x+ installed with idf.py on PATH (source export.sh), (2) USB-serial drivers for ESP32-S3 (CP210x or built-in USB-JTAG), (3) pyserial installed in the platform's Python environment. The flash endpoint validates these prerequisites before starting a build.

# 7. Directory Structure

The firmware stays in the repository but moves under a top-level firmware/ directory alongside the existing Hybrid/ and platform/ directories. The MQTT_TOF/espfix/dashboard/ is retired (replaced by platform frontend).

| Path | Notes |
| --- | --- |
| CAPSTONE_LOCALIZATION/ |  |
| ├── firmware/ | NEW top-level directory |
| │ ├── anchor/ | ESP-IDF project (from MQTT_TOF/espfix/anchor/) |
| │ │ ├── main/anchor_main.c | Anchor firmware (minimal changes) |
| │ │ ├── main/device_config.h | Auto-generated per-device config |
| │ │ └── CMakeLists.txt | ESP-IDF build config |
| │ ├── tag/ | ESP-IDF project (from MQTT_TOF/espfix/tag/) |
| │ │ ├── main/tag_main.c | Refactored thin-client firmware |
| │ │ ├── main/device_config.h | Auto-generated per-device config |
| │ │ └── CMakeLists.txt | ESP-IDF build config |
| │ └── common/ | Shared headers (MQTT helpers, JSON builder) |
| ├── Hybrid/ | Localization engine (unchanged path) |
| │ └── src_python/ |  |
| │ ├── engine_wrappers.py | + ToFWrapper, FusionWrapper |
| │ ├── data_pipes.py | Updated MQTTPipe subscription |
| │ └── app.py | + tof_decisions wiring |
| ├── platform/ | Web platform (unchanged path) |
| │ ├── backend/api/ |  |
| │ │ └── esp_flash.py | NEW: build + flash router |
| │ └── frontend/index.html | + ESP Flash panel in UI |
| └── MQTT_TOF/ | ARCHIVED (kept for git history, not used) |

# 8. Implementation Order

The refactoring is sequenced to maintain a working system at each step:

| Phase | Task | Risk | Dependency |
| --- | --- | --- | --- |
| 1 | Refactor tag firmware to thin client | Low | None |
| 2 | Switch anchor MQTT to JSON + new topics | Low | None |
| 3 | Update MQTTPipe subscription + parsing | Medium | Phase 1-2 |
| 4 | Implement ToFWrapper in engine | Low | Phase 3 |
| 5 | Wire tof_decisions into app.py + frontend | Low | Phase 4 |
| 6 | Build esp_flash.py backend module | Medium | Phase 1-2 |
| 7 | Add ESP Flash UI panel | Low | Phase 6 |
| 8 | Move firmware/ directory, archive MQTT_TOF/ | Low | Phase 1-7 |